MACHINA-X: Explainable AI-Driven Digital Twin Framework for Predictive Maintenance

Comprehensive Empirical Research Report & Algorithmic Benchmark Evaluation

| Research Project: | MACHINA-X Cyber-Physical Digital Twin Platform |
| --- | --- |
| Primary Domain: | Predictive Maintenance of Three-Phase Induction Motors & Industrial Equipment |
| Report Type: | Academic Technical Evaluation & Model Validation Report (TR-2026-MX1) |
| Evaluation Date: | October 2026 — Verified Model Checkpoints |

# 1. Executive Summary

This technical report presents the experimental validation and empirical benchmarks for MACHINA-X, an explainable digital twin framework designed for condition monitoring, multi-fault diagnostics, and remaining useful life (RUL) estimation in electro-mechanical machinery. The system integrates Motor Current Signature Analysis (MCSA), vibration signal decomposition, thermal telemetry, Gradient Boosted Decision Trees (XGBoost), Bidirectional LSTM networks, and TreeSHAP explainability into a coherent cyber-physical architecture.

The evaluation encompasses two distinct empirical tracks: (1) a benchmark on the UCI AI4I 2020 Predictive Maintenance dataset (10,000 physical instances), where the champion model achieves 98.45% accuracy and an F1-score of 0.7862 under extreme 28.5:1 class imbalance; and (2) an induction motor digital twin simulation modeling a 3 kW, 415 V motor under five operating states, attaining 94.55% classification accuracy and an RUL prediction error of 0.22 hours.

# 2. Cyber-Physical System Architecture

The MACHINA-X architecture is structured into five cohesive layers designed for high-throughput industrial telemetry:
• Ingestion Layer: High-speed ingestion of stator current (10 kHz), vibration (5 kHz), and surface temperature (1 Hz).
• Feature Extraction Engine: Computes 26 physics-guided features across spectral, time, and physical domains.
• Anomaly Detection: Unsupervised Isolation Forest filtering anomalous telemetry prior to classification.
• Fault Classification & Prognostics: Multi-class XGBoost classifier and sequence-based LSTM regression.
• Explainability Tier: TreeSHAP generates exact additive feature attributions for operators and engineers.

# 3. Track A Benchmark: UCI AI4I 2020 Dataset

The UCI AI4I 2020 dataset represents 10,000 real-world milling machine manufacturing cycles. It contains 339 failure events (3.39% incidence rate), presenting an authentic industrial class imbalance ratio of 28.5:1. An 80/20 stratified split with 5-fold cross-validation was employed to ensure zero data leakage and rigorous statistical validity.

Table 1: Comparative Algorithmic Benchmark on UCI AI4I 2020 Held-Out Test Set (N=2,000)

| Model Architecture | Accuracy | Precision | Recall | F1-Score | ROC-AUC |
| --- | --- | --- | --- | --- | --- |
| Logistic Regression (Baseline) | 85.75% | 17.80% | 88.24% | 0.2963 | 0.9393 |
| Decision Tree (Baseline) | 95.00% | 38.89% | 82.35% | 0.5283 | 0.8568 |
| Random Forest | 98.40% | 74.32% | 80.88% | 0.7746 | 0.9831 |
| XGBoost Classifier (Champion) | 98.45% | 74.03% | 83.82% | 0.7862 | 0.9808 |

Table 2: Confusion Matrix for Champion XGBoost (Held-out Test Split, N=2,000)

|  | Predicted Negative (Healthy) | Predicted Positive (Failure) |
| --- | --- | --- |
| Actual Negative (Healthy = 1,932) | 1,912 (True Negatives) | 20 (False Positives) |
| Actual Positive (Failure = 68) | 11 (False Negatives) | 57 (True Positives) |

Table 3: Global TreeSHAP Feature Attributions (AI4I Benchmark)

| Rank | Feature Variable | Mean Absolute SHAP Value |
| --- | --- | --- |
| 1 | Rotational_speed_rpm | 1.3314 |
| 2 | Tool_wear_min | 1.2755 |
| 3 | Mechanical_Power_W | 1.2263 |
| 4 | Overstrain_Metric | 0.5984 |
| 5 | Temp_difference_K | 0.5015 |
| 6 | Torque_Nm | 0.4996 |
| 7 | Process_temperature_K | 0.2971 |
| 8 | Air_temperature_K | 0.2453 |
| 9 | Type_Code | 0.0646 |

# 4. Track B Benchmark: Induction Motor Digital Twin

Track B models a 3 kW, 415 V, 50 Hz, 4-pole three-phase squirrel-cage induction motor operating under four load states (25%, 50%, 75%, 100%). Twenty-six features are extracted per 1-second analysis window (10,000 samples at 10 kHz). A multi-class XGBoost classifier classifies 5 distinct operating conditions across 312 test instances.

Table 4: Multi-Class Fault Classification Performance Across Operational States

| Fault Class | Precision | Recall | F1-Score | Test Samples |
| --- | --- | --- | --- | --- |
| HEALTHY | 76.92% | 41.67% | 0.5405 | 24 |
| BROKEN_ROTOR_BAR | 100.00% | 100.00% | 1.0000 | 72 |
| STATOR_INTER_TURN_SHORT | 100.00% | 100.00% | 1.0000 | 72 |
| BEARING_DEFECT | 83.13% | 95.83% | 0.8903 | 72 |
| AIR_GAP_ECCENTRICITY | 100.00% | 100.00% | 1.0000 | 72 |
| Overall Model (Macro Avg) | 92.01% | 87.50% | 0.8862 | 312 (Accuracy: 94.55%) |

# 5. Remaining Useful Life (RUL) Prognostics

To estimate remaining operating hours before threshold failure, a 2-layer Bidirectional LSTM network was evaluated over sliding windows of length k=5. The LSTM achieved a Mean Absolute Error (MAE) of 0.2199 hours (~13.2 minutes) and a Root Mean Squared Error (RMSE) of 0.2927 hours across 384 test sequences, demonstrating a 33.5% error reduction compared to the Ridge regression baseline (MAE: 0.3310 hours).

# 6. Limitations & Academic Integrity Statement

To uphold the highest standards of scientific rigor, the experimental scope is explicitly defined:
1. Benchmark Distinction: The UCI AI4I 2020 dataset is an established synthetic benchmark for milling machines and does not represent physical induction motor measurements.
2. Motor Telemetry Provenance: The 26 induction motor features are derived from physical mathematical models of 3 kW motors. They accurately model sideband harmonic dynamics and bearing vibrations but have not been recorded on a physical dynamometer testbed.
3. Deployment Readiness: Physical experimental testbed calibration is recommended before deploying the model in safety-critical installations.